An Even Easier Introduction to CUDA

Comprehensive Learning Notes

Muhammad Arham
Computer System Engineering Student
Dawood University of Engineering & Technology
March 4, 2026

1. Introduction and Resources

This notebook accompanies Mark Harris's popular blog post An Even Easier Introduction to CUDA.

For those just starting out, please Fundamentals of Accelerated Computing with CUDA Python., which provides:

- Dedicated GPU resources and sophisticated programming environment

- NVIDIA Nsight Systems™ visual profiler

- Dozens of interactive exercises and detailed presentations

- Over 8 hours of material

- DLI Certificate of Competency

2. Parallel Computing Fundamentals

2.1 What is Parallel Computing?

Parallel Computing is the simultaneous use of multiple compute resources to solve a computational problem. Instead of one processor working through a to-do list line-by-line, you break the task into smaller pieces that can be solved at the exact same time by multiple processors.

2.2 The Three Levels of Parallelism

1. Bit-Level Parallelism Increasing the processor word size (e.g., moving from 8-bit to 64-bit). This allows the hardware to process larger numbers of more instructions in a single cycle.

2. Instruction-Level Parallelism (ILP) The processor decides which instruction can run at the same time without changing the result (Pipelining is a classic example).

3. Data Parallelism (The CUDA Sweet Spot) This is where the same operation is performed on a large data set.

Example: Adding two vectors of 1 million numbers. In parallel computing, you don't loop 1 million times; you tell 1 million threads to each perform one addition simultaneously.

2.3 Why Parallel Computing is Hard: Amdahl's Law

As a developer you might think, "If I have 10 cores, my code will be 10x faster!" Unfortunately, it doesn't work that way because of Amdahl's Law.

Formula:

Where:

- p is the proportion of the program that can be made parallel.

- 1- p is the part that must remain serial (like loading a file or starting the OS)

Key Takeaway: If 10% of your code is strictly serial, your program can never be more than 10x faster, no matter how many thousands of GPU cores you throw at it.

3. Understanding CUDA

3.1 What is CUDA?

CUDA (Compute Unified Device Architecture) is more than just a library—it's a parallel computing platform and programming model developed by NVIDIA.

Think of CUDA as a bridge that allows you, the developers, to use high-level language like C++ to command the thousands of small, specialized cores inside a GPU.

3.2 The Core Philosophy: Latency vs. Throughput

To understand CUDA, you must understand why we use GPUs in the first place.

The CPU (The Generalist): Designed for low latency. It's like a high-speed Ferrari that can do one complex task at a time very quickly. It excels at branching logic (if/else) and sequential tasks.

The GPU (The Specialist): Designed for high throughput. It's like a massive fleet of 5,000 slow-moving trucks. A single truck is slower than the Ferrari, but together, they can move a mountain of data in one trip.

CUDA is the instruction set that tells those 5,000 trucks how to coordinate.

3.3 How CUDA Works: The Kernel

In standard programming, a function runs once. In CUDA, you define a Kernel. When you launch a kernel, you aren't calling the function; you are triggering a grid of thousands of threads that all run that same function simultaneously on different pieces of data.

This follows the SIMT (Single Instruction Multiple Threads) architecture.

3.4 The CUDA Software Stack

CUDA isn't just one thing: it's a three-layered cake:

Table

| Layer | Component | Description |
| --- | --- | --- |
| Top | Libraries | Pre-written math/AI functions (cuBLAS, cuDNN, Thrust). Most AI engineers live here. |
| Middle | The Language | CUDA C++, which adds keywords like __global__ and <<< >>> to standard C++. |
| Bottom | The Driver | The low-level interface that communicates directly with the GPU hardware (PTX). |

4. CUDA Programming Model

4.1 The Big Picture: Host vs. Device

In CUDA, your system splits into two parts:

Table

| Component | Description |
| --- | --- |
| Host | The CPU and its memory (RAM) |
| Device | The GPU and its dedicated video memory (VRAM) |

The Workflow: You write "Host code" (standard C++) to manage the high-level logic and "Device code" (kernels) for the heavy lifting.

4.2 Key Keywords & Syntax

To make the jump from CPU to GPU, you only need a few new tools:

Table

| Keyword | Purpose |
| --- | --- |
| __global__ | The "Kernel" specifier. It tells the compiler this function runs on the GPU but is called from the CPU. |
| <<< ... >>> | The Execution Configuration. This is how you launch the kernel and tell it how many threads to use. |
| cudaMallocManaged() | Allocates Unified Memory. This creates a single pointer accessible by both CPU and GPU (handles the data transfer for you). |
| cudaFree() | Frees the allocated Unified Memory. |
| cudaDeviceSynchronize() | Forces the CPU to wait for the GPU to finish its work before moving to the next line of code. |

5. Memory Allocation in CUDA: Unified Memory

To compute on the GPU, you need to allocate memory accessible by the GPU. Unified Memory in CUDA makes this easy by providing a single memory space accessible by all GPUs and CPUs in your system. To allocate data in unified memory, call cudaMallocManaged(), which returns a pointer that you can access from host (CPU) code or device (GPU) code. To free the data, just pass the pointer to cudaFree().

The "Even Easier" part of this tutorial relies on Unified Memory. In older CUDA versions, you had to manually cudaMemcpy data from RAM to VRAM. Now, cudaMallocManaged() creates a "shared" space.

6. Thread Hierarchy: Threads, Blocks & Grid

Think of GPU as a massive factory. In CUDA, we organize parallelism in three-tier hierarchy:

Table

| Level | Analogy | Description |
| --- | --- | --- |
| Thread | The Worker | The smallest unit. A thread executes your kernel code. |
| Block | The Team | A group of threads. Threads inside the same block can talk to each other and share fast local memory. |
| Grid | The Entire Factory | A group of blocks. All the blocks combined make up the "Grid", which represents the total work for one kernel launch. |

6.1 Scaling Parallelism: The Thread Hierarchy

This is where the magic happens. To process a million elements, you don't just loop; you launch a million threads.

6.2 Built-in Variables

The GPU gives every thread "ID cards" so it knows which piece of data to work on:

- threadIdx.x: The index of the thread within its block

- blockIdx.x: The index of the block within the grid

- blockDim.x: The number of threads in a block

6.3 The Indexing Formula

To find the unique global index of a thread in a 1D array:

index = blockIdx.x \times blockDim.x + threadIdx.x

6.4 From 1 Thread to Many

The tutorial progresses through three stages of kernel launches:

Stage 1: Serial

cpp

add<<<1, 1>>>(...)  // Runs once (useless for speed)

Stage 2: One Block, Many Threads

cpp

add<<<1, 256>>>(...)  // Limited to max threads per block (usually 1024)

Stage 3: Many Blocks, Many Threads (The Grid)

cpp

int blockSize = 256;

int numBlocks = (N + blockSize - 1) / blockSize;

add<<<numBlocks, blockSize>>>(N, x, y);

This is the production standard. It ensures you cover all N elements even if N isn't a multiple of 256.

6.5 Understanding the Launch Configuration

cpp

add<<<GridSize, BlockSize>>>(N, x, y);

- First Value (GridSize): The number of Blocks in the Grid

- Second Value (BlockSize): The number of Threads per Block

Important: When using add<<<1, 256>>>(N, x, y), you are using 256 threads. But if your kernel has a for-loop from 0 to N, every single one of those 256 threads will run the entire loop. You'll be doing the same work 256 times! (Race Condition).

The Goal: You want to eliminate the for-loop and let each thread handle just one index.

7. Streaming Multiprocessors (SM)

The Streaming Multiprocessor (SM) is the fundamental hardware unit in an NVIDIA GPU that executes your CUDA kernels. If the GPU is a "factory," the SMs are the individual "workshops" where the actual processing happens.

7.1 Key Components

Table

| Component | Function |
| --- | --- |
| CUDA Cores | The arithmetic units (ALUs) that perform integer and floating-point math |
| Registers | Extremely fast, private memory allocated to each individual thread |
| Shared Memory (L1 Cache) | A high-bandwidth, low-latency memory space shared by all threads within a single block |
| Warp Schedulers | The hardware logic that manages and issues instructions to groups of 32 threads (Warps) |
| Special Function Units (SFUs) | Specialized hardware for complex math like sines, cosines, and square roots |

7.2 Software-to-Hardware Mapping

CUDA follows a strict hierarchy when mapping your code to the physical silicon:

Table

| CUDA Software Concept | GPU Hardware Resource |
| --- | --- |
| Thread | CUDA Core |
| Warp (Group of 32 Threads) | Warp Scheduler |
| Block | Streaming Multiprocessor (SM) |
| Grid | The entire GPU (Device) |

7.3 Core Operational Principles

A. The Warp (SIMT)

The SM manages threads in groups of 32 called Warps. It uses SIMT (Single Instruction, Multiple Threads), meaning every thread in a warp executes the exact same instruction at the same time on different data.

B. Occupancy and Latency Hiding

Because accessing Global Memory (VRAM) is slow, an SM doesn't wait for data to arrive. Instead, the Warp Scheduler instantly switches to a different Warp that is ready to work. This "context switching" has zero overhead because registers are pre-allocated.

C. Block Residency

Once a Block is assigned to an SM, it stays there until every thread in that block has finished executing. An SM can host multiple blocks at once, provided it has enough Registers and Shared Memory to accommodate them.

7.4 Engineering Constraints (Why SMs Matter)

- Warp Divergence: If an if/else statement causes threads in a warp to take different paths, the SM must run each path sequentially, wasting performance.

- Resource Limits: If your kernel uses too many registers or too much shared memory per thread, fewer blocks can fit on the SM, leading to low Occupancy (idle hardware).

- Scalability: A GPU with more SMs will run a large Grid faster because it can process more Blocks in parallel.

8. Pascal Architecture

8.1 What is Pascal?

Pascal is the codename for NVIDIA's GPU architecture (the 10-series, like the GTX 1080 or the Tesla P100). It was designed specifically to bridge the gap between traditional graphics and high-performance computing (HPC) for AI and deep learning.

8.2 Key Architectural Innovations

For a Computer System Engineering student, these four features are the "DNA" of the Pascal architecture:

A. NVLink (High-Speed Interconnect)

Before Pascal, GPUs communicated with the CPU over the PCIe bus, which acted like a slow, narrow bridge. Pascal introduced NVLink, a high-speed "highway" that allows GPUs to talk to each other and the CPU up to 5x faster than PCIe.

B. HBM2 (High Bandwidth Memory)

Pascal moved away from traditional GDDR5 memory in its high-end chips (like the P100) to HBM2. By stacking memory chips vertically on the same package as the GPU, it achieved massive data throughput (>700 GB/s), which is vital for the parallel computing tasks you're studying.

C. Unified Memory (The "Even Easier" Feature)

This is the most relevant part for your current CUDA notes. Before Pascal, you had to manually move data between CPU and GPU. Pascal introduced Hardware Page Faulting.

- If the GPU tries to access data that is currently on the CPU, the Pascal hardware automatically "faults," pauses the thread, moves the data over, and resumes.

- This is why your code can use a single pointer for both x and y without manual cudaMemcpy calls.

D. 16-bit Floating Point (FP16)

Pascal was optimized for deep learning by allowing "half-precision" math. It can perform two FP16 operations in the same time it takes to do one FP32 operation. This doubled the throughput for AI training.

8.3 The GP100 SM Structure

In Pascal, the Streaming Multiprocessor (SM) was redesigned to be more efficient:

- Cores per SM: 64 CUDA cores (down from 128 in the previous Maxwell architecture)

- Benefit: By having fewer cores per SM but more SMs overall, the GPU could better manage the "registers" and "shared memory" for each core, leading to higher Occupancy (keeping the hardware busier)

8.4 Comparison: Maxwell vs. Pascal

Table

| Feature | Maxwell (Previous) | Pascal (Current Study) |
| --- | --- | --- |
| Process Node | 28 nm | 16 nm (Smaller/Faster) |
| Interconnect | PCIe 3.0 | NVLink |
| Memory Tech | GDDR5 | HBM2 / GDDR5X |
| Unified Memory | Software-emulated | Hardware-accelerated |

9. The GPU: Full System View

Since you've already looked at the SM (the workshop) and Pascal (the specific architecture), we can now zoom out to the "Full System" view.

Think of the GPU (Graphics Processing Unit) as a Massively Parallel Coprocessor. While the CPU is the "Manager" of the computer, the GPU is the "Engine Room" designed for one specific goal: performing the same mathematical operation on massive amounts of data at the same time.

9.1 The Core Philosophy: "Throughput" over "Latency"

This is the most important concept in Computer System Engineering:

- CPU (Latency Oriented): Designed to get one task done as fast as possible. It has giant caches and complex branch prediction to make sure the next instruction is ready immediately.

- GPU (Throughput Oriented): Designed to get thousands of tasks done at once. It doesn't care if one specific calculation takes a little longer, as long as 10,000 of them finish together.

9.2 How it Works: The Hardware Pipeline

When you run your add.cu code, here is the journey that happens inside the silicon:

A. The Command Processor (The Dispatcher)

The CPU sends a signal over the PCIe bus (or NVLink) saying, "Hey GPU, run this kernel!" The GPU's Command Processor reads the instructions and breaks your Grid into Blocks.

B. The GigaThread Engine (The Scheduler)

This is the "Brain" of the GPU. It looks at all the available SMs and distributes the blocks to them. If an SM is full, the GigaThread Engine keeps the extra blocks in a queue until an SM frees up.

C. The Execution (Inside the SM)

Once a block is on an SM:

- The Warp Scheduler takes 32 threads and tells them to execute the first line of your add function.

- The CUDA Cores perform the math (e.g., y[i] + x[i]).

- If the data isn't in the cache, the warp "stalls," and the scheduler instantly switches to another warp to keep the cores busy.

9.3 The Memory Hierarchy (The Data Flow)

A GPU works by moving data through different levels of speed and size. As a Solution Architect, you need to know where the data lives:

Table

| Level | Analogy | Size | Speed |
| --- | --- | --- | --- |
| HBM/GDDR (Global Memory) | The "Warehouse" | 8GB–80GB | Slowest |
| L2 Cache | The "Loading Dock" | Several MB | Fast |
| Shared Memory/L1 | The "Workbench" | 64-96 KB per SM | Very Fast |
| Registers | The "Tools in Hand" | KB per thread | Fastest |

9.4 Why it Works for AI (Agentic AI Connection)

Since you're working on Agentic AI, the GPU's work is essentially Matrix Multiplication.

- A "Thought" in an AI model is just a series of weights

- The GPU takes a vector of "input" and multiplies it against a matrix of "weights"

- Because every multiplication is independent, the GPU assigns one thread to every single multiplication. Instead of doing them one by one, it does billions of them in a single "heartbeat"

10. Summary: The GPU Workflow

- Copy data from CPU memory to GPU memory (or use Unified Memory)

- Instruct the GPU to launch a kernel with a specific Grid/Block geometry

- Execute in parallel across thousands of CUDA cores

- Synchronize (wait for the GPU to finish)

- Retrieve the result back to the CPU